# Commander Wi-Fi access (Vijeo Air app)

## Version date: 7/5/2018

## Warning

You must consider the potential of accidental touches from remote mobile devices.  Know and understand the hazards regarding equipment movement started by a remote operation.   You, as the operator, are responsible for all gates and interlock for protecting people and equipment.

Failure to follow these instructions can result in death, serious injury, or equipment damage.

### Required Hardware (source locally)

Wi-Fi router

Ethernet cable

Tablet or cell phone with Wi-Fi capability

### Hardware installation

Locate Wi-Fi router in a suitable location near the HMI enclosure.

The Wi-Fi router should be powered by its own power supply.

Connect an ethernet cable from Wi-Fi router to the ethernet switch inside the HMI enclosure.

### Router configuration

Configure router

static IP address 	10.10.11.5

subnet mask 		255.255.255.0

gateway 		10.10.11.211

Set router name (NECO dryer).

Set router security password to prevent unauthorized access.

Record security password here _______________________________________

### Required Software (download app)

Schneider Vijeo Design’Air (available on google apps)

### Tablet / Phone Wi-Fi settings

Connect to router (NECO dryer).

Enter router security password.

### Initial Vijeo Design’Air app settings

Press		“+ Add device” icon

Name: 		NECO dryer

Host:		10.10.11.101

Port:		37891

Press 		“OK”

### Using the Vijeo Air app

Note:  The use of this app allows remote access to monitor, make minor operational setting changes and to stop the dryer only.  Starting of equipment must be performed at the dryer HMI panel.

The local operator (User, Owner or NecoTech) via the HMI must start and get the dryer and associated equipment in the desire state of operation.

Once all equipment is running as desired, the local operator enables remote access

MAIN MENU > SETUP > SECURITY SETUP > I ACCEPT > ENABLE REMOTE ACCESS.

When the “ENABLE REMOTE ACCESS” icon is pressed the current operator is logged out and the “Remote” user is logged in on the HMI.

The user is now able to access the dryer control via the Vijeo Design’Air app.

Press the app icon on the tablet / phone, then press “NECO dryer”.

Read accept the liability statement.  Press “OK” or “View-only” icon.

Login.

Username:	RemoteWiFi

Password:	1379

Press		“OK”

At this point the tablet / phone displays the same image the HMI screen.

The operator may remotely

change Burner Temperature(s)

change Target Moisture

change Manual Metering Speed Setpoint

and view all screens

Note: If the “LOGOUT” icon is pressed the dryer control will not be able accessible until operator locally re-enables the remote access.