PLC Firmware loading

Turn off power—1CB

Insert the SD card

Turn power back on—1CB

initial lights

SD Light is on for Approx. 2+ minutes

ERR light comes on, loading is finished.

Turn power off 1CB, remove SD card

Turn Power back on 1CB

The initial lights

Then just then err light is on.

When the ETN (ethernet) light comes on and the err light blinks the PLC is ready for a program.